AO 9544 Aeolus + Innovation. SEA-FLECT: Winds from Aeolus LIDAR SEA Surface ReFLECTance. Final Report. Doc. No.:; AERD/P2466/ESA/AO9544/D10. Issue. No.:; 1. Version:; 1.0.0. Date:; 8 July 2022. Document prepared under ESA Contract Number: 4000133131/20/I-BG.

Document Change Record

| Issue / Revision | Date | Changed pages / paragraphs |
| --- | --- | --- |
| 1.0.0 | 08/07/2022 | Initial version. |

Table of Contents

1	Introduction	4

2	Science Requirement Baseline	5

2.1	Lidar return from surface range bin.	6

2.2	Experimental setup	6

2.3	Risk assessment	7

3	The DataPool	8

3.1	Aeolus data	8

3.2	Main dataset	9

3.3	Secondary dataset	9

4	Algorithm Theoretical Basis	10

5	Validation	12

5.1	Validation of AUX-MET surface wind speed.	12

5.2	Validation of the wind speed derived from Aeolus observations.	12

5.3	Sensitivity analysis	12

5.4	Validation of the Aeolus measurements	13

6	Summary and Conclusions	16

6.1	Summary	16

6.2	Conclusions	16

6.3	The Way Forward	17

6.3.1	Towards an improved Aeolus surface wind retrieval method	17

6.3.2	End of Life Experiment	17

7	Annex I: Surface Reflectance from GOME/Sciamachy	19

8	Deliverable Items	20

9	References	20

# Introduction

The objective of the SeaFlect project was to explore the potential to monitor the sea surface state using Aeolus data. In particular to document the potential to monitor the sea surface wind speed. To derive sea surface wind speed from lidar measurements is not new. For instance, Menzies, David and William, (1998) reported a study which involved the Lidar In-space Technology Experiment (LITE), while Li et al. (2010) used observations by the Aeolus Airborne simulator to explore the dependency of the observations with windspeed.

However, at time of writing no attempts have been made to explore the observations by the Aeolus mission with the objective to monitor sea surface wind speed. The main mission objective of Aeolus is to monitor the tropospheric wind. Thus, the SeaFlect project explores the Aeolus data beyond the main mission objectives, and it is not clear that the Aeolus data have the potential to be used in this way.

The project follows largely the approach documented by Li et al. (2010), with the exception that a classification method was developed to remove unsuitable Aeolus data from the analysis. Where Li et al. (2010) only considered the potential relation between surface wind speed and the ACCD counts, here a processor was developed to infer from the Aeolus ACCD counts the sea surface wind.

The current document provides a summary the various deliverables provided to the Agency:

- The Science Requirement Baseline (SRB) provides a description of the physical principles which are at the basis of the project, together with a description of the experimental setup. This document is summarized in section,

- The Auxilliary Data Document (AUX) provides a description of the datasets prepared for the project,

- The Algorithm Theoretical Basis Document (ATBD) provides a description of the geophysical model function (GMF) which was prepared and used to derive the windspeed from the Aeolus observations,

- The Validation Report (VAL) provides a detailed description of the analysis and results,

- The Science Roadmap (SRM) contains an extensive set of recommendations for future activities and also for future space borne instruments.

# Science Requirement Baseline

Figure 1: One full orbit of Aeolus observations (29.11.2021, starting 03:55:52 UTC) for the surface range bin (bin 23) of the parameter “Mie signal intensity”.

Figure 1 shows one full orbit (29.11.2021 starting 03:55:52 UTC) of the Aeolus L1B “Mie Signal Intensity” product for the surface range bin. No cloud screening has been applied. The following can be observed:

- Observations between ca. 70 and 150 are taken over Africa, with 70 near the Egyptian coast east of Alexandria and 150 on the western South-African coast.

- High Mie signal intensities at observation values between ca. 70 and 90 represent different arid surface types located in the Sahara Desert.

- Observation values from ca. 200-240 represent Antarctica, whereas observation values between ca. 420 to 440 represent the Arctic. This corresponds with high Mie signal intensities, as expected.

- After leaving the Antarctic, the orbit crosses the Pacific with hardly any land interaction. Observation values near 300 represent oligotrophic waters between French Polynesia and New Zealand.

The observed variations in the L1B Mie signal intensity appear plausible and justify the attempt to retrieve the surface wind speed from Aeolus surface range bin observations via a model relating instrumental output to wind speed.

## Lidar return from surface range bin.

The surface range bin is a composite range bin, consisting of an atmospheric and oceanographic sub-layer separated by the sea surface interface. The lidar return signal is result of different processes in this composite range bin. A schematic illustration of these processes is provided in Figure 2 and illustrates the complexity of the problem.

The return signal is generated by particle and molecular scattering processes in both the atmospheric and sub-surface sub-layer, in combination with reflectance from the surface interface.

Figure 2: Illustration of physical processes involved in the lidar return signal from the surface range bin.

## Experimental setup

The project aims to reduce the complexity by considering only lidar return signals free from particle scattering from the atmospheric sublayer. Furthermore, the molecular scattering from the atmospheric sub-layer is estimated, using the return signal from the range bin directly above the surface range bin. Left is then the return signal from the surface interface in combination with the return signal from the sub-surface layer. Specific regions are selected for which the sub-surface contribution to the return signal is assumed to be stable in space and time or low. This allows us to analyse the variation of the return signal as function of surface windspeed. In particular:

- Deep Blue Ocean: A region in the South Pacific Gyre was selected because of its low chlorophyll content resulting in  stable (but high) reflectance in the UV. The likelihood of high winds in this region is low.

- High Winds: In addition to the SPG region, a region in the Southern Hemispheric Storm Tracks was selected to analyse the return signal under high wind conditions. However, the sub-surface contribution is more variable than in the SPG.

- Dark Ocean: Three regions which have low reflectance in the UV were also selected. As the sub-surface contribution to the return signal is small, it would facilitate to analyse the return signal dependency on wind speed. On the downside, the geographical extend of these regions is small and suffer from high cloudiness.

For the regions two multiyear datasets were prepared. One covering the period September 2019 – September 2020, as Aeolus instrument was stable and good (cf. Figure 3). The second dataset covered the period September 2019 – March 2022. The first data set was primarily used to analyse the Aeolus observations, while the second one was used to analyse the measurements. It should be noted that the measurements represent the laser return signal with a spatial resolution of approximately 3 km, while the observations are an accumulation of 30 measurements and consequently represent the laser return signal with a spatial resolution of about 87 km.

Figure 3: The Normalised Aeolus internal and external signals over different paths and detectors since switch-over to the 2nd flight laser in July 2019. Violet: Output UV energy (~70 mJ). Yellow: EMT emission path. Green: internal INT reference path. Red: atmospheric ATM return path. Credit: O. Lux, DLR, Aeolus DISC consortium.

## Risk assessment

Based on the above considerations, a risk assessment was performed which indicated that in particular the low performance of the instrument, and the lack of a precise model for the return signal from the surface range bin were identified as high risk for the success of the project. More details are presented in [SRB].

# The DataPool

Two datasets were generated to support the analysis. For sake of completeness the Aeolus data is briefly described.

## Aeolus data

The Aeolus instrument package consists of the ALADIN laser and a detector sub-system. The ALADIN laser emits laser pulses at 355 nm, with a frequency of 50.5 Hz at an angle of 35o with respect to the sub-satellite point [(Reitebuch, Huber and Nikolaus, 2018), cf Figure 4]. The return signal received by the detector sub-system, records the signal using a detector array, which stratifies the signal into 24 range gates, corresponding to 24 layers in the Earth-Atmosphere system.

Figure 4: Basic observation geometry of the Aeolus mission. The geographical extend of the measurements is approximately 3 km (indicated by the purple rectangles), while that of the observations is approximately 87 km. (ESA).

The return signal is available to the users with two different geographical sampling. The data with the highest spatial sampling (approximately 2.9 km) are referred to as measurements and contain the accumulation of 20 pulses. To improve the signal to noise, the measurements are accumulated into what is referred to as observations. Nominally 30 measurements are used to generate one observation, however, the number of measurements per observation is a configurable parameter and was changed towards the end of 2021 and beginning of 2022 (https://www.aeolus.esa.int/confluence/display/CALVAL). At time of writing each measurement consists of 114 pulses (from 20), with a corresponding spatial scale of 18 km, and each observation consists now of 5 measurements.

## Main dataset

The main database, consisting of the Accumulated Charged Coupled Device counts (ACCD) from the Mie channel, was generated directly from the Aeolus data repository (ftp://aeolus-da.eo.esa.int) by a custom-made retrieval and processing script. The database consisted of the latest reprocessed Aeolus Level 1B data for the period September 2019 – September 2020. The Aeolus level 1B data was collocated with surface wind information extracted from the AUX MET data also available from the above-mentioned Aeolus data repository. Furthermore, ocean colour information collocated with the Aeolus data. This ocean colour data was extracted from the Mercator repository at https://marine.copernicus.eu.

Finally, the main database was complemented with selected products of the Aeolus Level 2A data stream.

HY-1C/HY-2B/CFOSAT data were obtained from https://osdds.nsoas.org.cn. The authors would like to thank NSOAS for providing the data free of charge. Only small amounts of data could be retrieved manually at each time. Hence this data is only available for a few months.

This main dataset was used to analyse a possible relationship between the Aeolus observations and surface wind speed, and to perform a limited validation of the surface wind speed provided by the AUX MET data.

## Secondary dataset

Near the end of the project the virtual research environment (https://VRE.aeolus.services) was made available. This environment was used to derive a secondary dataset consisting of Aeolus Level 1B data, which consisted of the ACCD-counts from the Mie channel, collocated with meteorological information extracted from the AUX MET data stream. This secondary dataset covered initially a period from September 2019 – March 2022.

However, because of changes in the instrument settings, resulting in a courser spatial resolution of the measurements after November 2021, only period from September 2019 – September 2021 was used. This secondary dataset was used to appropriate the quality and completeness of the Aeolus measurements, and to analyse possible relation between the Aeolus measurements and the surface wind speed.

# Algorithm Theoretical Basis

The Algorithm Theoretical Basis Document [ATBD], provides a description of the geophysical model function (GMF) adopted for the analysis. The GMF follows the theoretical model prepared by Li et al. (2010) and is repeated here for completeness:

Rmod=Rwc+ 1-WRs+ 1-RwcRu

In this equation the white cap fraction is indicated by W, and the reflectance by Rwc. The reflectance of the sub-surface by Ru, and the specular reflectance by Rs. The model assumes that the reflectance of white caps does not depend on the age of the white caps and that the scattering is isotropic in all directions. The dependence of the white cap fraction with wind speed is modelled using a function prepared by Monahan and O’Muircheartaigh (1980). Similarly, the back scatter from the sub-surface is also assumed to be isotropic and constant. Although there is a term describing the specular reflectance, its contribution is small due to the observation geometry (lidar incidence angle ~ 38° on the sea surface).

An illustration of the various terms for a typical situation is provided in Figure 5. Which shows that the variation of reflectance with windspeed is dominated by the white cap contribution.

Figure 5: Total reflectance according to the GMF. Also shown are the three contributing terms, the contribution from the white caps (green), underwater (blue) and specular reflectance (red)

The proposed retrieval methodology consists of a number of steps:

- Identify observations not affected by clouds or high aerosol loads using the signal-to-noise ratio (SNR) and scattering ratio (SCAT).

- Using ad-hoc scaling to convert the ACCD-counts into “reasonable” reflectance values.

- Estimate the atmospheric contribution to the surface range bin from the adjacent range bin above the surface range bin.

- Estimate the sub-surface contribution to the upward light field just above the sea surface as a function of chlorophyll-a concentration obtained from external sources (CMEMS operational ocean colour L3 product).

- Invert the GMF model to retrieve surface wind speed from observations.

The inversion search for a minimum in a cost function, where it is assumed that the error in the GMF model is the identity matrix. In particular let J be the cost function, calculated through:

J=RmodU,C-RobsTSmod-1RmodU,C-Robs

Where the modelled reflectance (Rmod) is a function of the wind speed (U) and the chlorophyll concentration (C). And the error covariance matrix (Smod) is assumed to be the identity matrix. The retrieved wind speed is the minimum value of the cost function for a given observation.

For the method to work, the Mie useful ACCD counts of the surface range bin, after removing the atmospheric contribution, are converted into reflectance values, using an ad-hoc linear scaling method, which uses the observed dynamical range of the ACCD counts and the observed dynamical range of the wind speed as provided by the AUX-MET data to estimate an offset and slope of this linear transformation.

# Validation

## Validation of AUX-MET surface wind speed.

The surface wind speed as provided by AUX-MET was validated using independent observations from the scatterometer on board the HY-2B platform. Only manual retrieval of small dataset from the Chinese repository was possible, such that only a limited validation was performed. This demonstrated the accuracy of the AUX-MET surface wind observations.

Figure 6: Comparison wind speed from AUX-MET and Aeolus observations with the wind speed from scatterometer on board HY-2B, for the South Pacific Gyre region, and September 2020 period. The AUX-MET data points are indicated by the blue dot, while the retrieved values by the orange asterisk.

Figure 6 shows the comparison for September 2020, between the surface winds as provided by the HY-2B scatterometer and the AUX-MET as blue dots.

## Validation of the wind speed derived from Aeolus observations.

After this the surface wind from the main observational dataset was derived. Which resulted in negative results. An example is also provided in Figure 6. It should be noted that the GMF indicates a relation in the strong wind regime (wind speed in excess of 10 m/s). Unfortunately, the number of Aeolus observations in this regime is relatively small and also the maximum wind speed observed is only 12 m/s. These limitations could explain part of the results.

## Sensitivity analysis

It was concluded that the proposed retrieval approach to Aeolus data did not result in satisfactory results. A few sensitivity studies were performed to better understand the dependencies between Aeolus observations and wind speed. This sensitivity study revealed a number of limitations with both data and the retrieval scheme.

- The L1B Mie signal intensity is only provided in instrument counts, no conversion into physical units is available.

  - An ad-hoc scaling has therefore been attempted, but this cannot replace a proper vicarious calibration. This could reduce the sensitivity of the observations to the wind speed, as it was assumed that the maximum return signal corresponds to the maximum wind speed, which was not demonstrated.

  - Calibration is important as this would facilitate the merging of the observations collected from different regions and under different sub-surface conditions.

- The noise in the L1B signal intensity is significant in relation to the signal expected from the sea surface, limiting the sensitivity of the proposed approach.

  - From the applied geophysical model, we estimate that a reflectance resolution on the order of 10 [%] is required to resolve wind speed at 1 m/s, but that this strongly depends on the adopted model to describe the white cap fraction as function of wind speed.

  - Due to the Aeolus observation geometry rendering specular reflection at the sea surface highly unlikely, foam coverage appears to be required to provide a measurable signal.

- It is difficult to separate between the atmospheric, sea surface, and sub-surface contributions to the surface range bin:

  - The geometrical extension of atmosphere and sub-surface inside the surface range bin are not known with high accuracy and was changing during the period of interest. Our understanding is that the height of the interfaces between the range bins has an error of 100m, which is significant for range bins considered here, which have a typical thickness of 400 m.

  - The estimation of the sub-surface contribution based on the Adam database did not improve, for unknown reasons.

- From the sensitivity analysis it can be argued that the various components of the GMF should be consolidated. The backscatter from the sub-surface and from the white caps is assumed to be isotropic which might be an accurate assumption. It is further assumed that the horizontal distribution of white caps is homogeneous, and that the radiometric properties of white caps is not time dependent. These assumptions should be consolidated with up-to-date publications, as it can be expected that the newly formed white caps will have a different structure than older white caps. Both effects, the non-homogeneous distribution of white caps, and the time variation of the white cap radiative properties will have a pronounced effect on our analysis.

## Validation of the Aeolus measurements

A main assumption of the adopted analysis is that the return signal from the surface range bin is free of scattering by particles and consists of only scattering by molecules. The observations have a spatial resolution of approximately 87 km, and it is unlikely that there are no clouds within such a large field of view. This could influence the analysis. To better select Aeolus data which are less likely affected by particle scattering, an analysis of the Aeolus measurements, with a spatial resolution of approximately 3 km was performed.

For this analysis data from the Aeolus repository using tools made available through the virtual research environment (vre.aeolus.services) were used, which also included methods to co-located the Aeolus measurements with the AUX-MET data. This analysis followed largely the analysis presented above, although some threshold settings were adopted, because of the different information content. Also the analysis considered a 2 year period from September 2019 – September 2021. Although also the period September 2021 – March 2022 was retrieved, as the number of laser pulses per measurements were changed in this period, resulting in a courser spatial resolution, this period was not considered for the analysis. The analysis also considered the three different regions mentioned above: the deep blue ocean, the high wind case and the dark ocean.

A quality and completeness check of the dataset indicated that there are no suitable measurements in the period September 2019 – October 2020. Requirements on SNR (between 10 – 20 for the surface range bin (range bin index 22) and the adjacent atmospheric range bin (with index 21) and low scattering ratio for range bin 21 (a scattering ratio of 1 indicates molecular scattering) removed all measurements for this period. Further inspection also revealed that the measurements collected in March 2022 were not suitable for the analysis. This resulted in a significant reduction of the meaningful datapoint. In total for the different regions, only 10.000 points were available for the analysis (out of approximately 2.000.000).

The analysis considered a correction of the return signal for the atmospheric contribution, a scaling of the range corrected ACCD counts to a parameter which has a dynamical range consistent with the theoretical results of the GMF and independent observations by e.g. GOME/SCIAMACHY ((Tilstra et al., 2016). The analysis did not try to invert the reflectances into a wind speed as was done for the observations.

A positive result was found for the Gulf of Lion. This is an area in the western part of the Mediterranean Sea East of Spain, where very strong fohn winds can be observed. The sub-surface contribution is moderate, such that the return signal of the surface range bin is more sensitive to changes at the surface.

Figure 7: Relation between proxy surface reflectance and surface wind speed for the Gulf of Lion for the period November 2020 - September 2021.

Results are shown in Figure 7, which shows the expected increase in reflectance with increasing wind speed for the high wind regime (wind speed in excess of 10 m/s). Because of the relatively small area, and the short time domain, only 150 valid data points were available for the region. The scatter in the low wind regime, might be a result of uncertainties in the analysis method, and uncertainties in the measurement.

A positive result was also found for the Hudson Bay, which is part of the Dark Ocean group. However, the data set is too small to draw any conclusions from this.

Results for the South Pacific Gyre (Deep Blue Ocean group) was not positive, where it should be noted the dynamical range of the observed wind speed is not as large as for the Gulf of Lion. Which means that it is not possible to analyse the measurements where the sensitivity of the return signal to wind-speed is largest. And also the results for the Southern Hemispheric Storm Track (part of the High Windspeed Group) were not positive, maybe because of the time variation of the sub-surface contribution.

# Summary and Conclusions

## Summary

The SeaFlect project considered the value of Aeolus data for the monitoring of sea state, in particular the potential to monitor high surface wind speed. This is important for applications which concern the safety-at-sea, climatology and air-sea fluxes among others. The project derived two data sets which were used for an analysis of the Aeolus return signal of the surface range bin at both observation and measurement horizontal scales (87 and 3 km respectively). The analysis of the observations considered data collected between September 2019 – September 2020, while for the analysis of the measurements, a longer period was adopted namely September 2019 – September 2021. The project considered three observation scenarios:

- Deep Blue Ocean, characterised by a stable but significant sub-surface contribution to the surface range bin return signal because of the extremely low chlorophyll content. The likelihood of finding Aeolus data points collected during extreme wind conditions is low in this scenario.

- High winds, characterised by a more variable sub-surface contribution to the return signal because of varying chlorophyll concentrations in the sea, but with a likelihood of finding Aeolus data points collected during extreme wind speeds.

- Dark Ocean, characterised by a low sub-surface contribution to the return signal because of the high concentration of UV absorbing substances in the ocean.

It was not possible to derive an adequate (i.e., validated) wind product from the analysis of the 87 km observations. The analysis of the 3 km measurements however, especially of the data collected over the Gulf of Lion (part of the High Winds scenario) indicated a sensitivity of the Aeolus measurement to wind speed in line with the theoretical results.

## Conclusions

In conclusion, it appears that there is a potential for establishing a high-wind speed measuring system which is of interest to the operational ocean community as some sensitivity evident in cloud-free regions could be established.

This should be interpreted in light of the following confounding factors:

- The primary mission of Aeolus is to support the monitoring of tropospheric winds. The Aeolus data provided to the users does not contain radiometrically calibrated products such as radiance or reflectance,

- Because of the experimental nature of the Aeolus mission, the quality and completeness of the surface range bin return signal might not have received the same attention as the data supporting the main mission, resulting in reduced number of adequate data samples for the present analysis,

- Vertical resolution may smear near-surface return signals in adjacent range bins, while at the same time the location of the range bin heights might be uncertain because of the finite time sampling of the return signal,

- Clouds and aerosols generally block surface signal, the use of the scattering ratio as a measure to determine surface range bins free of particle scattering requires consolidation,

- Subsurface reflection varies widely, the input data to describe the sub-surface reflectance is uncertain,

- Sensitivity to white capping is expected only at high wind speeds (> 12 m/s), while at the same time the theoretical model to describe the white cap contribution to the return signal requires a critical analysis, and an update in light of recent publications.

## The Way Forward

### Towards an improved Aeolus surface wind retrieval method

Not all relevant aspects and arising questions could be addressed in the context of the SeaFlect project. Based on experience gained during the project, a number of improvements to the geophysical model function, additional analyses are proposed to better understand the potential of Aeolus observations for the proposed applications. A detailed description is provided in the Scientific Road Map document. A key component is the application rigorous radiative transfer simulations to simulate the Aeolus observations under a diverse set of conditions to better understand if there is a reasonable expectation that Aeolus data can be used to monitor the sea surface state, and under which conditions this can be done. These calculations can pave the way to better understand the potential to radiometrically calibrate the Aeolus data. Because of the finite size of the light source, and the scattering processes considered here (scattering by molecules and particles) requires a Monte Carlo radiative transfer model, which can describe also the polarised state of the radiation field as Aeolus measures one component of the return signal polarised state.

### End of Life Experiment

The Aeolus laser light emitted under an angle of approximately 35o from nadir. It has been reported that the  return signal strength will increase with decreasing nadir angle.  For instance Menzies, David and William (1998) reported theoretical considerations, which indicate an increase of several orders of magnitude of the lidar sea surface reflectance for small nadir angles compared to an angle of 35o (cf. their figure 1, which is shown here as Figure 8).

Figure 8: Modeled lidar reflectance from sea surface for various wind speeds. The subsuface volume backscatter is not included. Source Menzies, David and William (1998) .

Aeolus is approaching its end of life and will be decommissioned soon. This opens the opportunity to explore the scientific-experimental nature of Aeolus mission beyond its main mission objectives. To collect data under a different viewing geometry for an extended period. In particular to make measurements under an angle closer to nadir as the high spatial resolution (3 km), for at least a two week period to collect enough cloud free measurements of the regions of interest adopted for the SeaFlect project. Such an experimental dataset would be of great interest to the project team, and would significantly add to the scientific value of the Aeolus mission.

# Annex I: Surface Reflectance from GOME/Sciamachy

Figure 9: Surface reflectivity climatologies from UV to NIR determined from Earth observations by GOME‐2 and SCIAMACHY. The grey lines indicate the standard deviation. Source:(Tilstra et al., 2016) Tilstra et al. [2017].

Figure 9 shows reflectance spectra for different surface type regions derived from SCIAMACHY observations. A number of aspects of relevance to the proposed activities can be drawn from those:

- Reflectance values on the order of 0.08 in the UV are confirmed for oligotrophic waters,

- Desert targets cannot be considered being bright at the target wavelength (actually, they are dark in the UV!),

- Both Greenland and Antarctica are suitable as bright targets in the UV,

- Vegetation is generally dark in the UV.

See (Tilstra et al., 2016) for further details.

# Deliverable Items

The list of deliverable items is presented in

| Item | Name | Version | Date | Comment |
| --- | --- | --- | --- | --- |
| D1 | Scientific Requirements Baseline | V 1.2.0 | 24-01-2022 |  |
| D2 | Data Pool |  |  | The co-located data in NetCDF format. |
| D3 | Dataset Description Document | V 1.1.0 | 16-03-2022 |  |
| D4 | Surface Wind Speed Product Algorithmic Theoretical Baseline Document | V 1.0.0 | 27-10-2021 |  |
| D5 | Validation Report | V 1.0.0 | 1-07-2022 |  |
| D6 | Web-pages |  |  |  |
| D7 | User Manual |  |  | Because no prototype product could be derived from the available data, it was considered not relevant to generate a user manual |
| D8 | Assessment Report |  |  | Because no prototype product could be derived from the available data, there is no assessment report. |
| D9 | Scientific Road Map | V 1.0.1 | 8-7-2022 |  |
| D10 | Final Report | V 1.0.0 | 11-7-2022 |  |

# References

Li, Z. et al. (2010) ‘Airborne Doppler LIdar Investigation of Sea Surface Reflectance at a 355-nm Utraviolet Wavelength’, J. Atmos and Ocean. Technol., 27, pp. 693–704. Available at: https://doi.org/2009JTECHA1302.1.

Menzies, R., David, T. and William, H. (1998) ‘Lidar In-space Technology Experiment measurements of sea surface directional reflectance and the link to surface wind speed’, Applied Optics, 37(24), pp. 5550–5559.

Monahan, E.C. and O’Muircheartaigh, I.G.O. (1980) ‘Optimal power law description of oceanic whitecap coverage dependence on wind speed’, J. Phys. Oceanogr., 10, pp. 2094–2099.

Reitebuch, O., Huber, D. and Nikolaus, I. (2018) ADM-Aeolus Algorithm Theoretical Basis Document (ATBD) Level1B Products. AE-RP-DLR-L1B-001. DLR, p. 117.

Tilstra, L.G. et al. (2016) ‘Surface reflectivity climatologies from UV to NIR determined from Earth observations by GOME-2 and SCIAMACHY’, J. Geophys. Res., 122, pp. 4084–4111. Available at: https://doi.org/10.1002/2016JD025940.

This data were obtained from by the Chinese Ocean Satellite Data Service Center (https://osdds.nsoas.org.cn). The authors would like to thank NSOAS for providing this data free of charge.